National University of Computer & Emerging Sciences, Karachi

FAST SCHOOL OF COMPUTING

Computer Organization & Assembly Language (EE 2003) Fall 2025

Assignment # 02                                                Deadline: 1st, Nov-2025

Q -1 :  What will be the final values in EAX, EBX, and the stack after the following instructions execute?

mov eax, 3

mov ebx, 7

push eax

push ebx

pop eax

push 10

pop ebx

pop eax

Q-2:  Implement the following pseudocode in x86 assembly language. All variables are 32-bit signed integers:

if (num1 > num2 && num3 != num4)

result = num1 + num3;

else if (num2 == num4)

result = num2 - num1;

else

result = 0;

Q-3 :  Implement the following pseudocode in x86 assembly language. All variables are 32-bit signed integers.

sum = 0; i = 0;

while (i < count)

{

if (array[i] > 0)

sum = sum + array[i];

else

sum = sum - 1;

i = i + 1;

}

Q-4 :  Consider the following code:

MOV AX, 0H

MOV ECX, 0AH

DOLOOP:

DEC AX

LOOP DOLOOP

What is the value of the AX register after completion of the DOLOOP?

Q-5 :  In the following code sequence, show the value of AL (in hexadecimal format only) after each shift or rotate instruction has executed:

MOV AL,0D4H

SHL AL,3 ;

MOV AL,0D4H

SAL AL,3 ;

STC

MOV AL,0D4H

ROL AL,1 ;

STC

MOV AL,0D4H

RCR AL,3 ;

Q-6 : Convert the following High Level Language code to equivalent assembly language:

While (op1 <= op2)

{

if (op1 >= x && op1 =< y)

{

z += 10;

}

else

{

z -= 10;

}

op1--;

}

Q-7: Write an assembly language program that checks whether a signed byte value in register AL is positive, negative, or zero,  and displays a message accordingly (use Irvine library procedures).

INCLUDE Irvine32.inc

Q-8: Write an assembly program that multiplies two 16-bit signed integers stored in variables num1 and num2, stores the 32-bit result in result, and displays it using WriteInt.

INCLUDE Irvine32.inc

Q-9: Implement the following C++ code in assembly language, using the flag registers(ZF and CF). Assume that all variables are 16-bit Un-signed integers:

int array[] = {10,60,20,33,72,89,45,65,72,18};

int sample = 50;

int ArraySize = sizeof array / sizeof sample;

int index = 0; int sum = 0;

while( index < ArraySize )

{

if( array[index] <= sample )

{ sum += array[index]; }

index++; }

Q-10 : In Assembly language, multiplication by constants can be optimized using shifts and additions, since any multiplier can be expressed as a sum of powers of 2.

Using this concept:

(a) Write an equivalent Assembly code sequence (without using the MUL instruction) to multiply the unsigned integer value in EAX by 26 using shift and addition instructions.

(b) If EAX initially contains 2, show the final value in EAX after executing your code.