Computer Organization and Assembly Language (EL-2003)

Semester: Fall 2025 Section: BCS-3J / BCS-3K Course Instructor: Ms. Seema Hashwani

LAB # 05

Working with Data related Operator and Directives, Addressing

# Lab Objectives

By the end of this lab, students will be able to:

- Analyze how arithmetic instructions (ADD, SUB) affect Flag Register flags (Zero, Carry, Sign, Auxiliary Carry, Parity, Overflow).

- Use direct-offset operands to access array elements in memory.

- Apply the OFFSET operator to get a variable’s memory address.

- Use the PTR operator to specify operand size (byte, word, doubleword).

- Determine variable size in bytes using the TYPE operator.

- Count array elements with the LENGTHOF operator.

- Calculate total array size in bytes with the SIZEOF operator.

- Access and modify data using indirect operands with registers.

- Use indexed operands with scale factors to access array elements.

Effect of Arithmetic Instructions on Flag Registers:

Status flags are updated to indicate certain properties of the result. Once a flag is set, it remains in that state until another instruction that affects the flags is executed

1) Z-Zero Flag:

The Zero flag is set when the result of an operation produces zero in the destination operand.

If the result is zero , ZF=1  Otherwise ,ZF=0

Example Source Code: include irvine32.inc

.data

.code

Main Proc mov AX,OFFFFH inc AX call dumpregs exit

Main ENDP

END main

Explanation:

FFFFh+1h = 10000h. Since we have 0000h in AX after increment, ZF is set.

Output Snap:

2) C-Carry Flag:

This flag is set, when there is a carry out of MSB in case of addition and borrow in case of subtraction.The Carry flag is set when the result of an operation generates an unsigned value that is out of range (too big or too small for the destination operand).

The INC and DEC (increment and decrement) instructions in assembly languages do not set the carry flag

1)Example Source Code:

include irvine32.inc

.data

.code

Main Proc mov AL,OFH add AL,OFIH call dumpregs exit

Main ENDP

END main

Explanation:

OFh+F1h = 100h. Since we have a carry, CF is set.

Output Snap:

2)Example Source Code:

include irvine32.inc

.data

.code

Main Proc mov AX,12AEH sub AX,12AFH call dumpregs exit

Main ENDP

END main

Explanation:

12AEh - 12AFh = -1h=FFFFh. Since it is negative (leading digits are 1), CF is set. Output Snap:]]

- S-Sign Flag:

This flag indicates the sign of the result of an operation. A 0 for positive number and 1 for a negative number.

- AC-Auxilary Carry Flag:

This flag is set, if there is a carry from the lowest nibble, i.e., bit three during addition, or borrow for the lowest nibble, i.e. bit three, during subtraction]]]] Suppose we add 1 to 0Fh. The sum (10h) contains a 1 in bit position 4 that was carried out of bit position 3:

For example                  mov al , OFh

Add al ,1                                            ; AC=1

- Parity flag :

Indicates even parity of the low 8 bits of the result, PF is set, if the lower 8 bits contain even number 1 bits. For 16- and 32-bit values, only the least significant 8 bits are considered for computing parity value.

26 (11010)———————— PF=0

102 (1100110)——————— PF= 1

Examples

Example 1: 8-bit result

Result = 1010 1100  (0xAC)

Number of 1s = 4  → Even

PF = 1

## Example 2: 16-bit result

Result = 1011 0011 0110 0101 (0xB365)

Least significant 8 bits = 0110 0101 (0x65)

Number of 1s = 3  → Odd

PF = 0

Example 3: 32-bit result

Result = 1001 0100 1110 0110 0011 1100 1010 0001

Least significant 8 bits = 1010 0001 (0xA1)

Number of 1s = 3  → Odd

PF = 0

6) O-Over flow Flag:

It indicates out-of-range result on signed numbers. Signed number are counterpart of the carry flag. The following code sets the overflow flag but not the carry flag.

mov AL,72H    	 	 	; 72H = 114D add AL,OEH  	 	 	; OEH = 14D sum is +128, since it is out of range of 8 bit reg, so overflow flag become 1.

Direct-offset Operands:

You can add a displacement to the name of a variable, creating a direct-offset operand Example:

.data

arrayB BYTE 10h,20h,30h,40h  arrayW WORD 100h,200h,300h

.code

mov al,arrayB ; AL = 10h  mov al,[arrayB+1] ; AL = 20h  mov ax,arrayW ; AX = 100h  mov ax,[arrayW+2] ; AX = 200h

Data Related Operators and Directives:

1. OFFSET operator:

The OFFSET operator returns the number of bytes between the label and the beginning of its segment.

.data  bVal BYTE ?  wVal WORD ?  dVal DWORD ?  dVal2 DWORD ?

If bVal is located at offset 00404000h, we would get:

mov esi, OFFSET bval  	; ESI = 00404000 mov esi, OFFSET wVal          ; ESI = 00404001 mov esi, OFFSET dVal  	; ESI = 00404003

mov esi, OFFSET dVal2         ; ESI = 00404007

2. PTR Operator:

Assembly instructions require operands to be the same size. However, it may be required at some point to operate on data in a size other than that originally declared. This can be done with the PTR operator (override a variable's default size).

For example, the PTR operator can be used to access the high-order word of a DWORD-size variable.

The syntax for the PTR operator is:  type PTR expression

Must be used in combo with a data type: BYTE, SBYTE, WORD, SWORD, DWORD, SDWORD, FWORD, QWORD, TBYTE.

.data

myDouble DWORD 12345678h

Suppose want to move 5678 to AX?

o Can NOT do mov ax,myDouble  	; error since mismatched sizes   Remember little-endian storage:

## Can use it to move smaller to larger too…

The DWORD PTR operator makes this possible:

.data

wordList WORD 5678h, 1234h

.code  mov eax, DWORD PTR wordList ; EAX = 12345678h\

3. TYPE Operator:

Returns the size, in bytes, of a single element of a variable.

Syntax: TYPE var_name

BYTE: 1  WORD: 2

Etc.

.data

v1 BYTE ? v2 WORD ?  v3 DWORD ?  v4 QWORD ?

TYPE v1  	;returns 1

TYPE v2  	;returns 2

TYPE v3  	;returns 4

TYPE v4  	;returns 8

Example

.data

var1 BYTE 20h  var2 WORD 1000h  var3 DWORD ?

var4 BYTE 10, 20, 30, 40, 50  msg BYTE ‘File not found’, 0

.code

| mov ax, type var1 | ; AX = 0001 |
| --- | --- |
| mov ax, type var2 | ; AX = 0002 |
| mov ax, type var3 | ; AX = 0004 |
| mov ax, type var4 | ; AX = 0001 |
| mov ax, type msg | ;AX = 0001 |

4. LENGTHOF Operator:

Returns number of elements in an array, base on values appearing on same line as its label

.data

byte1 BYTE 10,20,30  array1 WORD 30 DUP(?),0,0  array2 WORD 5 DUP(3 DUP(?))  array3 DWORD 1,2,3,4  digitStr BYTE "12345678",0

LENGTHOF byte1 	 	 returns 3

LENGTHOF array1  	 returns 30+2

LENGTHOF array2  	 returns 5*3

LENGTHOF array3  	 returns 4

LENGTHOF digitStr   	returns 9

Be careful:

myArray BYTE 10,20,30,40,50

BYTE 60,70,80,90,100  LENGTHOF myArray is 5   as opposed to  myArray BYTE 10,20,30,40,50,

60,70,80,90,100

LENGTHOF myArray is 10

5. SIZEOF Operator:

The number of bytes taken up by a structure, Basically:

LENGTHOF * TYPE

Example 1: SIZEOF Operator :

.data

intArray WORD 32 DUP(0)

.code

mov eax,SIZEOF intArray ; returns 64 = 32 * 2

Indirect Operands:

In 32-bit protected mode, an indirect operand can be any 32-bit general-purpose register (EAX, EBX, ECX, EDX, ESI, EDI, EBP, ESP) surrounded by brackets []. The register contains the address of some data, and the value is accessed using that address.

Example:

.data

byteVal BYTE 10h

.code

mov esi, OFFSET byteVal  	 	; ESI = address of byteVal  mov al, [esi]   	 	 	; AL = 10h .

- If the destination operand uses indirect addressing, a new value is placed in memory at the location pointed to by the register.

Example:

mov [esi], bl ; Store BL into memory at address ESI

- When incrementing memory values, the assembler must know the operand size.

Otherwise, it shows an error:

inc [esi] 	 	  	; ERROR: operand must have size

Solution: Use the PTR operator to specify size:

| inc BYTE PTR [esi] | ; Increment 1 byte |
| --- | --- |
| inc WORD PTR [esi] | ; Increment 2 bytes |
| inc DWORD PTR [esi] | ; Increment 4 bytes |

Arrays:

Indirect addressing is useful when working with arrays.

# 1)Array of Bytes

.data

arrayB BYTE 10h, 20h, 30h

.code  mov esi, OFFSET arrayB  	; ESI = address of first element

mov al, [esi]   	 	; AL = 10h  inc esi   	 	 	; Move to next element  mov al, [esi]   	 	; AL = 20h

# 2)Array of 16-bit Integers (WORD)

.data

arrayW WORD 1000h, 2000h, 3000h

.code  mov esi, OFFSET arrayW  	; ESI = address of first element  mov ax, [esi]   	 	; AX = 1000h

add esi, 2  	 	 	; Move to next element

mov ax, [esi]   	 	; AX = 2000h

# 3)Array of 32-bit Integers (DWORD)

.data

arrayD DWORD 11111111h, 22222222h, 33333333h

.code  mov esi, OFFSET arrayD  	; ESI = address of first element  mov eax, [esi]  	 	 ; EAX = 11111111h  add esi, 4  	 	 	; Move to next element  mov eax, [esi]  	  	; EAX = 22222222h

# IndexedOperands

## 1. Indexed Operands Overview

An indexed operand adds a constant to a register to generate an effective address.

Any 32-bit general-purpose register (EAX, EBX, ECX, EDX, ESI, EDI, EBP, ESP) can be used as an index register.

Syntax:

constant [reg32] [constant + reg32]

### Example 1: Indexed Operand with Byte Array

.data arrayB BYTE 20, 40, 60, 80

.code mov esi, 1

mov al, arrayB[esi]     ; AL = 40 inc esi

mov al, arrayB[esi]     ; AL = 60 mov esi, 3

mov al, [arrayB + esi]  ; AL = 80

## 2. Adding Displacements

Adding Displacements: The second type of indexed addressing combines a register with a constant offset. The index register holds the base address of an array.

### Example 2: Using Displacement with WORD Array

INCLUDE Irvine32.inc

.data

arrayW WORD 1000h, 2000h, 3000h

.code

main PROC mov esi, OFFSET arrayW

| mov ax, [esi] | ; AX = 1000h |
| --- | --- |
| mov bx, [esi+2] | ; BX = 2000h |
| mov cx, [esi+4] | ; CX = 3000h |

## 3. Scale Factors in Indexed Operands

When working with arrays of words or doublewords, the offset must consider the size of each element.

We use the TYPE operator to automatically calculate the correct offset.

Syntax:

constant [reg32 * TYPE constant]

### Example 3: Using TYPE Operator for WORD Array

INCLUDE Irvine32.inc

.data

arrayW WORD 1000h, 2000h, 3000h, 4000h

.code

main PROC

mov esi, 1

| mov ax, arrayW[esi * TYPE arrayW] mov esi, 2 | ; AX = 2000h |
| --- | --- |
| mov bx, arrayW[esi * TYPE arrayW] mov esi, 3 | ; BX = 3000h |
| mov cx, arrayW[esi * TYPE arrayW] | ; CX = 4000h |