| aQ | Computer Organization and Assembly Language (EL-2003) |
| --- | --- |
|  | Semester: Fall 2025 Section: BCS-3J / BCS-3K Course Instructor: Ms. Seema Hashwani |

LAB # 09

# Integer Arithmetic

## Lab Objectives:

By the end of this lab, students will be able to:

- Shift & rotate Instructions

- Multiplication and Division

- Extended Addition and Subtraction

# Shift and Rotate Instructions

The 8086-based processors provide a complete set of instructions for shifting and rotating bits.

- Shift Instructions: 
Shift instructions move bits a specified number of places to the right or left. The last in the direction of the shift goes into the carry flag, and the first bit is filled with 0 or with the previous value of the first bit.

# SHL Instruction

This instruction performs a logical left shift on the destination operand, filling the lowest bit with 0. The highest bit is moved to the Carry flag, and the bit that was in the Carry flag is discarded.

Syntax : SHL destination, count

The following lists the types of operands permitted by this instruction:

SHL reg, imm8 
SHL mem, imm8 
SHL reg, CL 
SHL mem, CL

Example: 
mov bl, 8Fh					; BL=10001111b 
SHL bl, 1 					; CF=1, BL=00011110b

mov al, 10000000b 				; AL=10000000b
SHL al, 2  					; CF=0, AL=00000000b

Bit Multiplication Example:

SHL can perform multiplication by powers of 2. Shifting any operand left by n bits multiplies the operand by 2 n .

For example, shifting the integer 5 left by 1 bit yields the product of  5 x 21 = 10

mov dl, 5 					;DL=00000101b 			=5 
SHL dl, 1 					;CF=0, DL=00001010b 		=10

# SHR Instruction

The SHR (shift right) instruction performs a logical right shift on the destination operand, replacing the highest bit with a 0. The lowest bit is copied into the Carry flag, and the bit that was previously in the Carry flag is lost.

Examples:

mov al, 0D0h 					; AL = 11010000b 
shr al, 1 					; AL = 01101000b, CF = 0

mov al, 00000010b 
shr al,2 					; AL = 00000000b, CF = 1

Bitwise Division:

Logically shifting an unsigned integer right by n bits divides the operand by 2n . In the following statements, we divide 32 by 21, producing 16.

mov dl, 32 			;DL=00100000b 			=32 
SHR dl, 1 			;DL=00010000b, CF=0 		=16

# SAL and SAR Instructions

The SAL (shift arithmetic left) instruction works the same as the SHL instruction. 
The SAR (shift arithmetic right) works like:

The following example shows how SAR duplicates the sign bit. AL is negative before and after it is shifted to the right:

mov al, 0F0h 				; AL = 11110000b (-16)
sar al,1 				; AL = 11111000b (-8), CF = 0

Sign division:

mov dl, -128 				; DL = 10000000b 
sar dl, 3 				; DL = 11110000b

Sign-Extend AX into EAX:

mov ax, -128 				; EAX = ????FF80h 
shl eax, 16 				; EAX = FF800000h 
sar eax, 16 				; EAX = FFFFFF80h

# Rotate Instructions

Rotate instructions also move bits a specified number of places to the right or left. For each bit rotated the last bit in the direction of the rotate operation moves into the first bit position at the other end of the operand. With some variations, the carry bit is used as an additional bit of the operand. RCR (Rotate Carry Right) and RCL (Rotate Carry Left) instructions carry values from the first register to the second by passing the leftmost or rightmost bit through the carry flag.

# ROL Instruction

The ROL (rotate left) instruction shifts each bit to the left. The highest bit is copied into the Carry flag and the lowest bit position. The instruction format is the same as for SHL:

Example:

mov al, 40h 				; AL = 01000000b 
rol al, 1 				; AL = 10000000b, CF = 0
rol al, 1 				; AL = 00000001b, CF = 1
rol al,1					; AL = 00000010b, CF = 0

Exchanging Groups of bits:

You can use ROL to exchange the upper (bits 4-7) and lower (bits 0-3) halves of a byte. For example, 26h rotated 4 bits in either direction becomes 62h.

mov al, 26h 
rol al, 4 				; AL = 62h

# ROR Instruction

The ROR (rotate right) instruction shifts each bit to the right and copies the lowest bit into the Carry flag and the highest bit position.

Example:

mov al, 01h					; AL = 00000001b  
ror al, 1 					; AL = 10000000b, CF = 1
ror al, 1					 ; AL = 01000000b, CF = 0

# RCL Instruction

The RCL (rotate carry left) instruction shifts each bit to the left, copies the Carry flag to the LSB, and copies the MSB into the Carry flag:

Example:

clc 						;CF=0
mov bl, 88h 					; CF=0 , BL=10001000b
rcl bl, 1					; CF=1 , BL= 00010000b

# RCR Instruction

The RCR (rotate carry right) instruction shifts each bit to the right, copies the Carry flag into the MSB, and copies the LSB into the Carry flag

Example:

stc 						;CF=1 
mov ah, 10h 					; AH =  00010000, CF = 1 
rcr ah, 1 					; AH =  10001000, CF = 0

# Applications

- Binary Multiplication

.code 								
mov eax, 123 
mov ebx, eax 
shl eax, 5 			; mult by 25 
shl ebx, 2 			; mult by 22 
add eax, ebx 			; add the products

- Isolating Data Fields

The following code example extracts the day number field of a date stamp integer by making a copy of DL and masking off bits not belonging to the field:

# SHLD Instruction

The SHLD (shift left double) instruction shifts a destination operand a given number of bits to the left. The bit positions opened up by the shift are filled by the most significant bits of the source operand.

Format:

SHLD reg16, reg16, CL/imm8 
SHLD mem16, reg16, CL/imm8 
SHLD reg32, reg32, CL/imm8 
SHLD mem32, reg32, CL/imm8

Example:

.data 
a WORD 9BA6h
.code 
mov ax, 0AC36h 
shld a, ax, 4 				; a = BA6Ah

# SHRD Instruction

The SHRD (shift right double) instruction shifts a destination operand a given number of bits to the right. The bit positions opened by the shift are filled by the least significant bits of the source operand.

Example:

.code 
mov ax, 234Bh 
mov dx, 7654h 
shrd ax, dx, 4 			;ax=4234h

# MUL Instruction

The MUL instruction is for unsigned multiplication. Operands are treated as unsigned numbers. The three formats accept register and memory operands, but not immediate operands. The Carry flag is clear (CF = 0) because AH (the upper half of the product) equals zero.

Syntax:

MUL reg/mem8 
MUL reg/mem16 
MUL reg/mem32

The table represents MUL operands:

EXAMPLE # 01:

INCLUDE Irvine32.inc 
.code 
main PROC 
mov eax, 0 
mov ebx, 0 
mov al, 5h 
mov bl,10h 
mul bl 				; AX = 0050h, CF = 0
call crlf 
call dumpregs 
exit 
main ENDP 
END main

EXAMPLE # 02:

.data 
val1 WORD 2000h 
val2 WORD 0100h 
.code
mov ax, val1 			; AX = 2000h
mul val2 			; DX:AX = 00200000h, CF = 0

EXAMPLE # 03:

mov eax, 12345h 
mov ebx,1000h 		; EDX:EAX = 0000000012345000h, CF = 0
mul ebx

# IMUL Instruction

The IMUL instruction is for signed multiplication. Operands are treated as signed numbers and result is positive or negative depending on the signs of the operands.

The x86 instruction set supports three formats for the IMUL instruction: one operand, two operands, and three operands.

- One-Operand Formats:

IMUL reg/mem8 		; AX = AL * reg/mem8
IMUL reg/mem16 		; DX:AX = AX * reg/mem16
IMUL reg/mem32 		; EDX:EAX = EAX * reg/mem32

- Two-Operand Formats:

IMUL reg16, reg/mem16 
IMUL reg16, imm8 
IMUL reg16, imm16

- Three-Operand Formats:

IMUL reg16, reg/mem16, imm8 
IMUL reg16, reg/mem16, imm16 
IMUL reg32, reg/mem32, imm8 
IMUL reg32, reg/mem32, imm32

Example:

The following instructions multiply 48 by 4, producing +192 in AX. Although the product is correct, AH is not a sign extension of AL, so the Overflow flag is set:

mov al, 48 
mov bl, 4 
imul bl 			;AX = 00C0h, OF = 1

The following instructions multiply -4 by 4, producing -16 in AX. AH is a sign extension of AL so the Overflow flag is clear:

.code 
main PROC 
mov eax, 0 
mov ebx, 0 
mov edx, 0 
mov ax, -2 
mov bx, 4 			; EDX:EAX = FFFFFFF8h, OF = 0
imul bx 
call crlf 
call dumpregs

The following instructions demonstrate two-operand formats:

Example :

INCLUDE Irvine32.inc 
.data 
word1 SWORD 4 
dword1 SDWORD 4 
.code 
main PROC 
mov eax, 0 
mov ebx, 0 
mov ax, -4 			; AX = -4
mov bx, 2			; BX = 2
call dumpregs 
imul bx, ax 			;BX = -8
call dumpregs 
imul bx, 2 			;BX= -16
call dumpregs 
imul bx, word1 		;BX= -64
mov eax, -16 
mov ebx, 2 
call dumpregs 
imul ebx, eax 
call dumpregs 
imul ebx, 2 
call dumpregs 
imul ebx, dword1 
call dumpregs 
exit 
main ENDP 
END main

The following instructions demonstrate three-operand formats: 
Example:

INCLUDE Irvine32.inc 
.data 
word1 SWORD 4 
dword1 SDWORD 4 
.code 
main PROC 
mov ebx, 0 
imul bx, word1, -2 
call dumpregs 
imul ebx ,dword1, -5 
call dumpregs 
exit 
main ENDP 
END main

# DIV Instruction

The DIV (unsigned divide) instruction performs 8-bit, 16-bit, and 32-bit unsigned integer division. The single register or memory operand is the divisor. The formats are

DIV reg/mem8 
DIV reg/mem16
DIV reg/mem32

The following table shows the relationship between the dividend, divisor, quotient, and remainder:

Example:

mov ax, 0083h 		; dividend 
mov bl, 2 			; divisor
div bl 				; AL = 41h, AH = 01h 
mov dx, 0 			; clear dividend, high
mov ax, 8003h 		; dividend, low
mov cx, 100h 			; divisor
div cx 				; AX = 0080h, DX = 0003h

# Sign Extension Instructions(CBW,CWD,CDQ):

Dividends of signed integer division instructions must often be sign-extended before the division takes place. Intel provides three useful sign extension instructions: CBW, CWD, and CDQ.

The CBW instruction (convert byte to word) extends the sign bit of AL into AH, preserving the number’s sign. In the next example, 9Bh (in AL) and FF9Bh (in AX) both equal −101 decimal:

Example:

.data 
byteVal SBYTE -101 			; 9Bh 
.code 
mov al, byteVal 			; AL = 9Bh 
cbw 					; AX = FF9Bh

The CWD (convert word to doubleword) instruction extends the sign bit of AX into DX:

.data 
wordVal SWORD -101 		; FF9Bh 
.code 
mov ax, wordVal 			; AX = FF9Bh
cwd 					; DX:AX = FFFFFF9Bh

The CDQ (convert doubleword to quadword) instruction extends the sign bit of EAX into EDX:

.data 
.code  
dwordVal SDWORD -101 		; FFFFFF9Bh
mov eax, dwordVal 
cdq					; EDX:EAX = FFFFFFFFFFFFFF9Bh

# IDIV Instruction

The IDIV (signed divide) instruction performs signed integer division, using the same operands as DIV.

Example: 
The following instructions divide -48 by 5.

.data 
byteVal SBYTE -48 			; D0 hexadecimal
.code 
mov al, byteVal 			; lower half of dividend
cbw 					; extend AL into AH
mov bl, +5 				; divisor
idiv bl					; AL= -9, AH= -3

# ADC Instruction

The ADC (add with carry) instruction adds both a source operand and the contents of the Carry flag to a destination operand.

Syntax: 
ADC Destination, source 
ADC reg, reg 
ADC mem, reg 
ADC reg, mem 
ADC mem, imm 
ADC reg, imm

Example # 01:

mov dl, 0 
mov al, 0FFh 
add al, 0FFh 			; AL = FEh
adc dl, 0			; DL/AL = 01FEh

# SBB Instruction

The SBB (subtract with borrow) instruction subtracts both a source operand and the value of the Carry flag from a destination operand.

Syntax: SBB Destination, source

Example:

mov edx, 7		; upper half 
mov eax, 1		; lower half 
sub eax, 2 		; subtract 2
sbb edx, 0		; subtract upper half